Time coverage analysis of lake

12/9/2025

SUMMARY

Each lake is going to have under 2 seconds inf not less for an image event to occur. Based on this simple simulation, we cover around 0.19 square km a second, this is disregarding any slewing and pointing and purely at a fixed angle.

BACKGROUND / PURPOSE

This is to address the speed at which we cover a single lake.  This could determine what camera we pick for the push broom setup.

SIMULATION SETUP

Software Used:

ANSYS STK 13

Model Parameters:

| Parameter | Value | Notes / Comments |
| --- | --- | --- |
| Altitude | 500 km | Max height we can reach |
| Payload Vertical Half Angle | 6.84 degrees | Average seems to be around this size |
| Payload Horizontal Half Angle | 0.5 degrees | Any smaller and the simulation breaks and doesn’t work |
| Main Target | Big_Buttonwillow_Lake_197 | Lake picked for smaller size |
| Area of Target Lake | 0.3 sq km | Small lake |
| Inclination | 99 degrees | SSO orbit param |
| Right Angular of Ascension Node (RAAN) | 3 degrees | Not uber necessary |
| Eccentricity | 0 degrees |  |

Boundary Conditions:

None

Assumptions:

This is assuming a fixed angle with a slant. So no slewing or pointing.

PROCEDURE

- Ansys STK

- Insert Satellite

- Insert Sensor

- Apply parameters for the satellite and sensor

- Insert place object for area around Merced https://data-cdfw.opendata.arcgis.com/datasets/CDFW::california-lakes-1/explore?location=36.729820%2C-119.022227%2C6.57

- Fix angle of satellite so it is facing lakes at a given time period

- Do the access graph/report

- Compute access

- Time should be around 1.5-2 seconds

RESULTS

Duration with lake in view of payload = 1.573 seconds

ANALYSIS

Without any slewing we are capable of covering both a large area of lakes. Timing is quick so the camera should support fast capturing of data.

CONCLUSIONS

Other parameters needs to be taking into account, for example jitter, slewing, GPS geolocation and more. This, however, gives a good idea as to how long we have to take an image without any slewing required.

REFERENCES

- None

### Last Known Author(s)

Name: Zander Abid

Email: zabid@ucmerced.edu

Date last Edited: 12/9/2025